A Physics-Encoded Neural Operator with Conformal Trajectory Bands for Fast Transient Stability Screening

Zhenyu Liu a,*, Jiale Wang a, Jiayong Liu a, Zhicheng Zeng a

a International Energy College, Jinan University, Guangzhou, 510632, China

Abstract—Transient stability assessment (TSA) predicts whether a power system remains synchronized after a large disturbance and estimates the critical clearing time (CCT) of faults. Time-domain simulation (TDS) is accurate but slow, whereas machine-learning classifiers are fast but discard the rotor-angle trajectory and provide no error bound. We introduce a physics-encoded neural operator mapping a fault scenario to the full center-of-inertia rotor-angle trajectory, with a distribution-free, finite-sample error band attached through split conformal prediction. The key enabler is a physics-informed input encoding: a fault is represented by the vector of initial accelerations it induces on each machine, computed from the swing equation rather than a bus index, letting the operator generalize to unseen fault locations and N−1/N−2 topologies. On the IEEE 39-bus system it classifies stability with 95.4% accuracy under a max-over-time π-separation criterion, estimates the CCT to within 25.2 ms, and runs ≈214​× faster than TDS on identical hardware. A directly calibrated pairwise score bounds the machine-angle spread to 148​∘ at α=0.1; the band is coverage-valid, but the screening rule then resolves none of the scenarios, so the resolvable fraction is reported across α with a TDS fallback—the conformal output is a calibrated trajectory band, not a stability certificate. Retrained on the IEEE 118-bus system it reaches 92.9%% accuracy, and the encoding carries over unchanged to higher-fidelity generator models. A physics-residual loss does not help, and generalization across operating points remains open. To our knowledge, this is the first work coupling a trajectory-level operator with conformal trajectory bands and CCT extraction in TSA.

Index Terms—conformal prediction, critical clearing time, neural operator, physics-informed machine learning, power system dynamics, transient stability.

Introduction

Transient stability is the ability of a power system to remain in synchronism after a large disturbance such as a three-phase fault. Its assessment is a safety-critical, recurring task: for a given contingency and fault clearing time, system operators must know whether the system survives and, ideally, how much clearing-time margin remains before it loses synchronism [1], [2]. The standard tool is time-domain simulation (TDS), which integrates the nonlinear swing equations and is accurate but computationally expensive, particularly when screening many fault locations, clearing times, and topology changes [3].

This cost has motivated a large body of machine-learning surrogates for TSA: classifiers such as support vector machines, decision trees, or deep networks [4] that map operating-condition or fault features to a binary stable/unstable label. Fast as they are, these surrogates discard the rotor-angle trajectory (and thus any stability margin or CCT estimate), do not generalize naturally across fault locations (a one-hot bus index yields no signal for unseen buses), and output no uncertainty or error bound, so an operator cannot know when the surrogate is trustworthy.

We address all three limitations with a physics-encoded neural operator equipped with conformal trajectory bands. The operator is a Deep Operator Network (DeepONet) [5], whose branch encodes a fault scenario and whose Fourier-feature trunk encodes time; their contraction yields the full center-of-inertia (COI) rotor-angle trajectory rather than merely a label. The fault scenario is encoded by a physics-informed vector: the initial acceleration (Pmi−Peif)/Mi that the fault induces on each machine, computed directly from the swing equation, which allows the operator to generalize across unseen fault locations. Split conformal prediction [6] attaches a finite-sample, distribution-free error band to every predicted trajectory, together with a tighter score that directly bounds the pairwise angle difference entering the stability criterion. Throughout, “physics-informed” refers to the input encoding derived from the swing equation, not to a residual-constrained model, and the conformal output is a calibrated trajectory band, not a stability certificate.

Our contributions are as follows.

- We couple, to our knowledge for the first time, a trajectory-level neural operator with conformal trajectory bands for rotor-angle trajectories and with CCT extraction. Prior neural operators for power systems target component-level dynamics [7]; conformalized operator regression has appeared for post-fault bus-voltage trajectories [8], targeting voltage rather than rotor-angle stability and without CCT extraction; and prior conformal work on TSA predicts only a binary stability label [9]. Section 2 positions these works relative to ours, and the “first” claim is scoped to rotor-angle trajectory bands with CCT extraction.

- We show that a physics-informed input encoding—the swing-equation acceleration vector—is what enables generalization across unseen fault locations, and we quantify the difference against a one-hot encoding and against graph baselines using raw observables.

- We provide a split-conformal calibration layer that yields a finite-sample, distribution-free error band on the trajectory and a directly calibrated pairwise-angle bound below the π threshold at α=0.1, together with the resolvable fraction, the wrong-release rate, and a CCT estimate compared against classical direct criteria; the derived CCT band is a heuristic interval without a formal guarantee (Section 4).

- We extend the encoding with a post-fault severity vector and show that a single operator generalizes to N−1 and N−2 line-trip topologies not seen in training.

- We report trajectory accuracy together with an ablation of an auxiliary physics-residual loss, including a negative result for this system and protocol: physics is injected more reliably through the input encoding than through a residual loss.

Related Work

Neural operators and physics-informed learning in power systems

Neural operators learn maps between function spaces rather than between fixed-size vectors; the dominant architectures are DeepONet [5] (branch–trunk contraction) and the Fourier Neural Operator [10]. Physics-informed neural networks (PINNs) [11] embed governing-equation residuals into the training loss.

Recent work has begun applying these tools to power systems. Karampinis et al. [7] propose a DeepONet-based framework for modeling power-system components, targeting fast dynamic simulation and digital twins, but do not address TSA, the CCT, or uncertainty quantification. Moya et al. [12] attach split-conformal intervals to DeepONets (Conformalized-DeepONet) for operator regression on canonical dynamical systems, and physics-informed graphical surrogates have been applied to power flow and optimal power flow [13], [14]. Our work instead targets the post-fault rotor-angle trajectory operator and couples it with conformal certification.

Learning-based transient stability assessment

Learning-based TSA has long been dominated by classifiers that predict a stable/unstable label from hand-crafted features [4]; more recent graph models with distribution-aware pooling [15] report high accuracies on the IEEE 39-bus system under in-sample or per-sample protocols. Under our held-out-bus protocol with the same raw features, however, a standard graph convolutional classifier reaches only 76.0% and a stronger residual graph model 77.3% (Section 6), and graph models produce no trajectory, CCT, or uncertainty bound.

A few works predict the post-fault trajectory rather than only the label; these are closest in spirit to ours, but they provide no error bounds. Event-structured PINNs model the pre-fault, fault-on, and post-fault stages with hard state chaining and a differentiable CCT boundary [16]; their error-propagation theorem is complementary to our finite-sample conformal band. PINN-based acceleration of time-domain simulation [17] trains a network with physics residuals, whereas we learn the scenario-to-trajectory map with a physics-informed input encoding; fast CCT calculation for mixed synchronous/asynchronous generation [18] is complementary to our TDS-based reference.

Confidence-aware TSA has begun to emerge. Lu et al. [9] combine a large language model with conformal prediction to produce prediction sets for a binary stability label, but they predict neither the trajectory nor the CCT and do not use a physics-informed operator. The closest works to ours are the conformalized operator regression of Mollaali et al. [8], which predicts post-fault bus-voltage trajectories without rotor angles, CCT extraction, or topology generalization, and the Conformalized-DeepONet framework of Moya et al. [12], which establishes operator-regression bands on canonical dynamical systems. Probabilistic and Bayesian operator frameworks (DeepONet-grid-UQ) [19] assume Gaussian or posterior-sampling uncertainty, and risk-controlling quantile operators (UQNO) [20] learn a second operator for pointwise radii; our split-conformal bands are distribution-free and require only a calibration set on top of the trained operator.

Across the axes that matter for TSA—physics-derived input encoding, unseen fault-bus and topology generalization, the calibration unit, and CCT extraction—no single prior work combines a physics encoding that generalizes across fault locations and topologies, a directly calibrated bound on the pairwise spread entering the stability criterion, and CCT extraction.

Classical direct methods remain strong CCT baselines: the transient-energy-function (TEF/PEBS) method [21] and the individual-machine equal-area criterion (IMEAC) [22] extract the CCT from machine-level energy criteria (Section 6). Our contribution is the combination of a trajectory-level neural operator with a physics-informed fault encoding and a conformal error band, which yields a CCT estimate alongside the trajectory; the derived CCT band is heuristic and carries no formal coverage guarantee (Section 4).

Problem Formulation

Classical multi-machine model

We consider the classical model of transient stability, in which each synchronous generator is represented as a constant voltage source Ei behind a transient reactance xdi′ [1], [2]. Under the standard reduction, loads are converted to constant admittances and the network is Kron-reduced to the internal generator nodes, yielding an n×n admittance matrix Y. The dynamics of generator i obey the swing equations

dδidt&=ωs(ωi−1),dωidt&=Pmi−Pei(δ)−Di(ωi−1)2Hi,

where δi is the rotor angle, ωi the per-unit speed, Hi the inertia constant, Di the damping coefficient, Pmi the mechanical power, and ωs the synchronous speed. The electrical power is

Pei(δ)=j=1nEiEjGijcos(δi−δj)+Bijsin(δi−δj),

with Gij=ℜ{Yij} and Bij=ℑ{Yij}. We use the undamped case Di=0, which is standard for the classical model and gives the undamped (energy-margin) stability boundary; a damped variant is examined in Section 6. Because the equations depend only on angle differences, it is convenient to work in the center-of-inertia (COI) frame, defining

δi=δi−δCOI,  δCOI=i​Miδii​Mi, Mi=2Hi.

A three-phase fault at a bus is modeled by adding a large shunt admittance to that bus and reducing the network during the fault; the fault is cleared after a clearing time tc, optionally tripping a line (an N−1 contingency). Reference trajectories integrate [eq:swing1]–[eq:swing2] with a fourth-order Runge–Kutta scheme at a 0.5 ms step, splitting each step at the clearing instant tc so the fault-on network is used exactly until tc (event alignment); the state is recorded on a 0.01 s output grid. A trajectory is stable if the maximum pairwise angle difference remains below π at every output-grid point over the simulation horizon—the π-separation criterion, the operational label used throughout the paper. It applies to the selected model, horizon, and criterion and is not claimed to coincide with infinite-horizon synchronism for arbitrary systems (Section 7).

Problem statement

Let a fault scenario be described by a fault location (bus), an optional set of tripped lines, and a clearing time tc. We seek the map G:(scenario)↦{δi(t)}i=1n, t∈[0,T], returning the COI-referenced rotor-angle trajectory, together with (i) a stability classification under the π-separation criterion, (ii) the critical clearing time tCCT, the first clearing time at which the criterion is violated as tc increases from zero—a dense sweep found a single dominant stable-to-unstable crossing on most buses (1–5 local flips at the 5 ms resolution, concentrated on the two largest-CCT buses), so on the held-out buses this coincides with the largest stable tc—and (iii) a calibrated error band on the trajectory, in the statistical sense made precise in Section 4.

Method

Physics-informed fault encoding

A central obstacle to generalization across fault locations is that a bus index is an arbitrary label with no physical meaning: replacing it with a one-hot vector gives the branch network no information about physically similar buses. We instead encode a fault at bus k by the vector of initial accelerations it induces, computed from the swing equation at the pre-fault equilibrium,

ai(k)=Pmi−Peif(k)Mi,

where Peif(k) is the electrical power of machine i during the fault at bus k, evaluated at the pre-fault angles—precisely the initial rate of change of per-unit speed in [eq:swing2] (the angular acceleration is ωsai). Dividing by Mi makes machines of widely different inertia contribute comparably, and continuity in the fault admittance maps physically similar faults to nearby encodings. The branch input is the concatenation of a(k)∈Rn and the normalized clearing time. The encoding is not claimed to be injective—two scenarios with similar encodings can still differ in later response near the stability boundary—but Section 6 shows it transfers the information needed for this task, while the conformal band flags the scenarios where the prediction is least reliable.

For N−1 contingencies, in which a line is tripped at fault clearing, the post-fault reduced admittance and the post-fault dynamics both change. We encode this change symmetrically with a post-fault severity vector,

ai(l)=Pmi−Peipost(l)Mi,

where Peipost(l) is the post-fault electrical power at the pre-fault angles with line l removed; for the intact network ai=0. The full scenario encoding concatenates the fault-on severity [eq:severity], the post-fault severity [eq:postseverity], and the clearing time. For N−2 contingencies the same vector is evaluated with both lines removed: a is always an n-dimensional vector evaluated at the pre-fault angles, so the encoding dimension is unchanged and the same branch network applies without modification.

Neural operator architecture

We use a DeepONet. The branch network bθ maps the scenario encoding to a matrix of basis coefficients B∈Rn×p; the trunk network τϕ maps time to p basis functions. The predicted trajectory is the contraction

δ(s,t)=B(s) τ(t)∈Rn.

Because the swing dynamics are oscillatory, we feed the trunk a Fourier feature embedding of time, {cos(2πkt/T),sin(2πkt/T)}k=1K, before a small multilayer perceptron, so the trunk represents the natural oscillation modes more efficiently than a plain perceptron. The trunk is smooth in t, so its time derivatives are available by automatic differentiation for the optional physics residual below. Fig. 1 summarizes the proposed pipeline.

Fig. 1.  Overview of the proposed physics-informed neural operator (N−0 configuration shown; N−1/N−2 additionally use the post-fault severity vector): a fault scenario is mapped through the physics-informed encoding to a branch network, whose coefficients contract with a Fourier-feature trunk to yield the predicted rotor-angle trajectory δ; split conformal prediction then attaches a distribution-free error envelope.

Physics-residual loss (auxiliary)

Following the PINN paradigm, we optionally add a residual of the post-fault swing equation in the COI frame. Differentiating the COI definition under Di=0 gives Miδi=ωs(Pmi−Pei)−Miωsc(t), where c(t)=j​(Pmj−Pej)/Mtot is the COI acceleration and Mtot=j​Mj. Dividing by Mi so that machines of widely different inertia contribute comparably (otherwise the H=500 s aggregate machine at bus 39 dominates the sum), the per-machine residual enforced at a set C of collocation times in the post-fault region t≥tc is

Lphys=1|C|t∈C, i​[δi−ωs(Pmi−Pei(δ))/Mi+ωsc(t)]2.

The per-machine division by Mi makes the term comparable across machines; it retains acceleration units and is a soft penalty rather than a hard constraint. The ωs factor enters through the frequency-to-angle relation and fixes the scaling between the curvature and power-mismatch terms. The total loss is the data loss plus λLphys, enforced only in the post-fault region (the fault-on admittance varies per scenario) at every fifth output time point; denser collocation is ablated in Section 6.

Conformal calibration

The trajectory operator alone gives no indication of its own error. We use split conformal prediction [6] to attach a finite-sample, distribution-free band to the single-machine COI angles, calibrated on stable scenarios: unstable trajectories diverge and carry errors orders of magnitude beyond any useful bound, so the band is defined on the stable subpopulation by design—a scope choice, not a claim that unstable dynamics are mathematically ill-posed. For a scenario s, the nonconformity score is the worst-case prediction error over machines and output-grid time points,

R(s)=maxi,t∈Tgrid|δi(t;s)−δi(t;s)|.

We split the N stable scenarios of the held-out test set (Section 5)—which the operator never saw during training and which are drawn at random from the same held-out-bus sampling distribution, so calibration and evaluation scores are exchangeable—into a calibration half (Ncal) and a disjoint evaluation half (Neva). For target miscoverage α, the bound q is the k-th order statistic of the calibration scores with k=⌈(Ncal+1)(1−α)⌉, the standard finite-sample split-conformal rule. For a new scenario exchangeable with the calibration scenarios and drawn from the same distribution conditionally on being truly stable, the whole predicted trajectory is then covered simultaneously on the output grid: Pr{maxi,t∈Tgrid|δi(t)−δi(t)|≤q∣scenario stable}≥1−α, i.e., with probability at least 1−α over exchangeable stable scenarios, every single-machine COI angle at every output-grid point lies within q of the truth. Three limits are explicit: the guarantee is marginal over the stable subpopulation (it does not cover misclassified unstable scenarios); it applies to the finite output grid (a denser-grid check found no missed criterion crossings, Section 6); and it is a calibrated trajectory band, not a stability certificate. By the triangle inequality, the pairwise angle difference relevant to the stability criterion is bounded by 2q, which at α=0.1 exceeds the π threshold.

To bound the decision quantity directly, we also calibrate a pairwise-difference score over the signed per-machine errors,

Rpw(s)=maxi,j,t∈Tgrid|ei(t)−ej(t)|,  ei(t)=δi(t)−δi(t),

which scores exactly the pairwise angle spread appearing in the π-separation criterion; the errors ei keep their sign, so that oppositely signed machine errors add rather than cancel. As shown in Section 6, Rpw bounds the pairwise difference to 148​∘ at α=0.1—below the π threshold, but too wide to certify decisions in practice: the rule spread+qpw<π resolves 0% of the predicted-stable cases at α=0.1, rising to 31% at α=0.3 and 67% at α=0.5 as the bound tightens. The pairwise bound is therefore a screening aid used with a TDS fallback for the unresolved fraction, not a stand-alone decision rule. Multiplicity-corrected per-machine bounds (Bonferroni/Šidák) do not tighten the pairwise bound (Section 6); any joint guarantee would additionally require the structural conditions of the chosen correction.

A CCT band can be derived from the per-machine interval: clearing times whose predicted trajectory, shifted by ±2q in spread space, cannot reach the π threshold are declared stable with margin, and clearing times whose shifted spread exceeds π for all shifts are declared unstable; the remainder forms the band, found by the same binary search with the shifted criterion. Because the trajectory bound at one tc does not transfer across tc (the CCT is a nonlinear functional of the trajectory), this CCT band is heuristic: it carries no formal coverage guarantee and is reported as a screening aid (Section 6).

Critical clearing time estimation

Given the operator, the CCT of a fault is estimated by binary search over the clearing time: for each candidate tc, the operator predicts the trajectory, and the scenario is declared unstable if the maximum pairwise angle spread exceeds π at any output-grid point—the same max-over-time criterion used for the reference labels. The search uses 16 iterations over [0.01,0.8] s ([0.005,0.2] s for the 118-bus system); the same protocol is applied to the TDS reference, so the two CCT estimates are directly comparable. The 12 μs interval is the search tolerance of the bisection, not the physical accuracy of the CCT; the reference itself uses the event-aligned RK4 scheme of Section 3, whose convergence was verified at a 0.25 ms step, and both judge stability on the 0.01 s output grid.

Experimental Setup

Test system and data

We use the IEEE 39-bus New England system with 10 machines, whose static data are the standard published values and whose dynamic parameters (H, xd′) are the canonical classical-model values [1], [21]; the specific parameter values and their source are tabulated in the code repository. The reference simulator integrates [eq:swing1]–[eq:swing2] with the event-aligned RK4 scheme of Section 3 at a 0.5 ms step over a 4 s horizon. We draw a fault bus uniformly over all 39 buses and a clearing time uniformly in [0.03,0.55] s, producing approximately 40% stable scenarios. The dataset has 2160 training, 540 validation, and 960 test scenarios; the test set is constructed from 12 fault buses held out entirely from training, so test performance measures generalization to unseen fault locations, while the validation set is drawn from the training buses. Event-aligned labels differ from the pre-alignment reference (which kept the fault on until the first grid point after tc) on exactly one of 960 test and one of 2160 training scenarios, both at the stability boundary. The CCT search range [0.01,0.8] s covers the observed 39-bus CCTs (0.13–0.58 s over all buses, 0.13–0.45 s on the held-out buses).

Baselines and ablations

We compare against the same DeepONet with (i) a one-hot fault encoding; (ii) a physics-residual loss at several weights; (iii) a self-attention branch; and (iv) a Fourier-neural-operator trunk, plus a pointwise multilayer perceptron (MLP) baseline mapping (scenario, time) to the angle directly, graph convolutional networks (GCNs) that predict the stability label or the CCT from raw node features (voltage magnitudes and angles, loads, and a fault flag), and a recurrent (GRU) sequence-to-sequence baseline. The CCT is additionally compared against the transient-energy function (PEBS) method [21] with the conductance path-integral terms of Athay et al., and an IMEAC-style individual-machine criterion [22]. The DeepONet variants, MLP, and GRU share the same dataset, optimizer, class-balanced mean-squared loss (saturating beyond ±3π), and 600-epoch budget; the GCNs use binary cross-entropy (or MSE) and 300 epochs on raw observables, so the GCN comparison measures the value of the severity encoding. A severity-MLP baseline receives exactly the same encoding as the operator, and the pointwise MLP and GRU are equipped with the same conformal calibration and CCT search, so all trajectory-level models are compared under one protocol; parameter counts and training budgets are in the repository.

Evaluation

We report the root-mean-square error (RMSE) of the predicted trajectory over stable test scenarios, decomposed into the fault-on (t<tc) and post-fault (t≥tc) regions; the stability classification accuracy under the max-over-time π-separation criterion, with the confusion matrix and precision, recall, and F1 of the stable class; the conformal band half-width and its empirical coverage with counts and binomial intervals; the CCT error against the event-aligned TDS binary search; and the wall-clock speedup over the RK4 reference. “RMSE” refers to the pooled RMSE over all stable scenarios, machines, and time steps in the relevant region, computed in one pass; it is dominated by near-boundary scenarios and is not comparable to a per-scenario time-averaged mean absolute error. All main-table numbers are exported by a single evaluation script from one model and one dataset; four-seed statistics are reported separately where available.

The operator is implemented in PyTorch (2.13, CUDA 12.6) and trained on a single NVIDIA RTX 4070 GPU; CPU timings use a single thread of an AMD Ryzen 7 7700. Both networks use tanh activations: the branch has three hidden layers of width 512 and outputs n×p basis coefficients with p=512; the trunk has three hidden layers of width 128 on top of K=16 Fourier harmonics. We train with Adam (learning rate 10−3, cosine schedule, 600 epochs, batch size 128). The main results (Table 1) are for the seed-0 model under the max-over-time criterion; the same script also exports the confusion matrix, and across four training seeds the classification accuracy is 94.0%±1.2% and the per-scenario mean RMSE is 20.2∘±1.4∘, so the reported values are representative. All test metrics other than conformal coverage (RMSE, classification, CCT) are computed on the full 960-scenario held-out set; only conformal coverage is restricted to a disjoint evaluation half (Section 4). The speedup is reported under two separate protocols on identical CPU hardware, with single-threaded implementations, warm-up, and median/P95 statistics (details in the repository). Throughput: over repeated batches of 64 scenarios the operator evaluates a 401-point trajectory at 0.131 ms per scenario against 28.0 ms for the RK4 reference (0.5 ms step, 4 s horizon), a ≈214​× gap. End-to-end latency: a single fault scenario, including the severity encoding (Kron reduction, ≈0.13 ms per bus on the 39-bus system) and the sequential 16-iteration CCT binary search, takes 30.6 ms versus 9735 ms for the same brute-force search with TDS, a 318× gap; a 100-point clearing-time sweep costs 12.3 ms end-to-end versus 2.3 s for RK4. On the 118-bus system the encoding (≈4.1 ms per fault) dominates a single scenario. The RK4 cost scales inversely with the reference step size, whereas the operator’s cost depends only on the number of output time points (401 per trajectory), so the speedup grows for finer reference steps; all timing details are in the repository (https://github.com/La0Fe1/operator-transient-stability).

Results

Trajectory prediction and classification

Table 1 summarizes the main results. The physics-informed operator predicts the fault-on segment of the trajectory to 2.3​∘ pooled RMSE and the post-fault segment to 36.9​∘ (full-horizon 36.3​∘); the post-fault error is concentrated near the stability boundary—the sensitivity the conformal band is designed to capture—with 10.5∘ in the first-swing window and 41.8∘ in the subsequent oscillation. The operator classifies stability under the max-over-time π-separation criterion with 95.4% accuracy and confusion matrix (TP, FP, FN, TN) =318, 6, 38, 598 (F1 for the stable class 0.935), and estimates the CCT to within 25.2 ms on average (max 204.9 ms on bus 12), at a ≈214​× throughput speedup over the RK4 reference.

For reference, the transient-energy-function (PEBS) approach of Athay et al. [21] with the conductance path-integral terms attains a mean CCT error of 18.7 ms with a conservative bias, and an IMEAC-style individual-machine criterion [22] yields 9.6 ms. The operator’s 25.2 ms mean absolute error is larger than both and carries a conservative bias (−21 ms), dominated by bus 12 (reference CCT 453 ms, estimated 248 ms), where the binary search lands between multiple local stable-to-unstable crossings of the operator’s criterion. Classical direct methods remain the more accurate CCT estimators; the operator’s value is that it additionally returns the full trajectory and a conformal band without any fault-on or post-fault integration. Fig. 2 shows predicted versus true COI trajectories for a representative near-boundary scenario.

TABLE I

Main results on the IEEE 39-bus system.

| Metric | Value |
| --- | --- |
| Stable trajectory RMSE (fault-on region) | 2.3​∘ |
| Stable trajectory RMSE (post-fault region) | 36.9​∘ |
| Stability classification accuracy (max-over-time) | 95.4% |
| precision / recall / F1 | 0.981 / 0.893 / 0.935 |
| CCT error (mean ¦err¦ / max) | 25.2 / 204.9 ms |
| Conformal band half-width (q, α=0.1) | 110.1​∘ |
| Conformal empirical coverage | 0.882 (157/178) |
| Pairwise bound (Rpw, α=0.1) | 148​∘ |
| Speedup vs. RK4 (CPU, throughput) | ≈214​×​ |

Fig. 2.  Predicted (dashed) versus true (solid) COI rotor-angle trajectories for the two most affected machines in a representative near-boundary stable scenario (fault bus 11, tc=0.1700​ s); the shaded band is the first-swing conformal band (70.8​ at α=0.1). The fault-on segment (t<tc, marked) is reproduced accurately, while the post-fault oscillation carries the bulk of the error. The first-swing window is defined by the true trajectory (an offline diagnostic); the per-machine band does not by itself certify the pairwise stability criterion.

Effect of the physics-informed encoding

Table 2 reports the ablation on the fault encoding. Replacing the physics-informed acceleration vector with a one-hot bus index degrades the stable-trajectory RMSE from 36.3​∘ to 141.4∘ and the classification accuracy from 95.4% to 88.0%, because a one-hot encoding provides no signal for unseen fault buses; the one-hot baseline also overfits at p=512—its RMSE exceeds the 110.1​∘ attained by a uniformly random predictor in [−180∘,180∘] on the same stable test trajectories, an actual baseline computed on the test set—and reducing its capacity does not recover meaningful generalization. The physics-informed encoding is therefore the mechanism enabling generalization across fault locations.

A GCN classifier operating on raw node features (voltage magnitudes and angles, loads, and a fault flag) achieves only 76.0% accuracy versus 95.4% for the severity-encoded operator; a stronger graph baseline with three residual layers and concatenated mean/max/sum pooling attains 77.3%±3.9% (five seeds), so the gap is not an artifact of a weak graph architecture, and a GCN regressing the CCT from the same raw features attains a mean CCT error of 48.0 ms (max 224.4 ms) versus 25.2 ms (max 204.9 ms). These comparisons are not architecture-controlled (the GCNs output a label or a scalar, the operator a trajectory), but they show the severity encoding carries more information than raw observables. A severity-MLP receiving exactly the same encoding attains 93.0% accuracy and 47.0 ms mean CCT error (Table 2): the encoding, not the DeepONet architecture, is the dominant factor for generalization to unseen fault locations. A recurrent (GRU) sequence-to-sequence baseline mapping the same encoding through a sequential decoder attains 57.3∘ RMSE and 93.9% accuracy, worse than the operator’s 36.3​∘: the low-rank operator structure outperforms a generic recurrent decoder.

12−TABLE II

Comparison of variants and baselines on the IEEE 39-bus system (12 held-out fault buses, seed 0). RMSE is the pooled stable-trajectory RMSE; “−” marks a metric the variant does not produce. The pointwise MLP and GRU are evaluated under the same criterion, conformal calibration, and CCT search as the operator.

| Variant | RMSE | Acc. | CCT err. |
| --- | --- | --- | --- |
| DeepONet, physics encoding (proposed) | 36.3​∘ | 95.4% | 25.2 ms |
| DeepONet, one-hot encoding | 141.4∘​ | 88.0%​ | −​ |
| DeepONet + physics-residual loss (λ=0.001) | 39.2∘​ | 95.6%​ | −​ |
| DeepONet + physics-residual loss (λ=0.01) | 76.5∘​ | 89.9%​ | −​ |
| Pointwise MLP baseline | 43.1∘​ | 96.9%​ | 20.7 ms |
| GRU sequence baseline | 57.3∘​ | 93.9%​ | 33.5 ms |
| Severity-MLP classifier (same encoding) | −​ | 93.0% | −​ |
| Severity-MLP CCT regressor (same encoding) | −​ | −​ | 47.0 ms |
| GCN classifier (raw features) | −​ | 76.0%​ | −​ |
| GCN CCT regressor (raw features) | −​ | −​ | 48.0 ms |

Generalization to N−1 and N−2 topologies

We train a single operator (p=512) on a mix of N−0 and N−1 scenarios (9990 scenarios, line trips drawn from 25 training lines) using the extended encoding of Section 4, and evaluate on two held-out axes: unseen fault buses and unseen trip lines (10 lines). On held-out line trips the operator achieves 33.6∘ stable-trajectory RMSE and 95.9% accuracy, demonstrating that the post-fault severity encoding lets the operator generalize to topologies never seen in training; on held-out fault buses the RMSE is 64.3∘ and the accuracy 93.5% (Table 3), reflecting the harder combined task. The same encoding extends to N−2 contingencies with no architecture change: on held-out line pairs the operator attains 43.4∘ RMSE and 92.9% accuracy, because the post-fault severity vector is simply evaluated from the admittance matrix with both lines removed. The multi-topology operator also estimates the CCT on held-out combinations: 19.4 ms mean absolute error on unseen trip lines, 31.9 ms on held-out fault buses, and 18.7 ms on held-out N−2 line pairs (sampled); split-conformal coverage on these splits is reported in Section 6.

N−0N−1TABLE III

Generalization of the multi-topology operator (N−0 + N−1 training): pooled stable-trajectory RMSE, classification accuracy, and CCT error (mean over held-out bus–line combinations).

| Held-out split | RMSE | Acc. | CCT err. |
| --- | --- | --- | --- |
| Trip lines (N−1, unseen) | 33.6∘​ | 95.9%​ | 19.4 ms |
| Fault buses (mixed N-0/N-1) | 64.3∘​ | 93.5%​ | 31.9 ms |
| Line pairs (N−2, unseen) | 43.4∘​ | 92.9%​ | 18.7 ms |

Sample efficiency of the operator

A motivation for the operator architecture is its low-rank inductive bias. Training the DeepONet and the pointwise MLP baseline on 10%, 25%, 50%, and 100% of the training scenarios (three seeds each), the operator attains 56.1∘ RMSE versus 65.2∘ for the MLP at 10% of the data (a ≈9∘ advantage), ≈8∘ at 25%, and ≈2–3∘ at 50%/100%. The low-rank structure therefore confers a modest sample-efficiency advantage in the low-data regime; with three seeds per fraction and a 14.0∘ across-seed spread at 10% data, the advantage is indicative rather than statistically established. These numbers use the base p=128 architecture (Table 2 reports p=512), which explains the different full-data values.

Extensions to higher-fidelity generator models and damping

To verify that the approach carries over to more realistic generator representations, we retrain the operator—architecture, encoding, and protocol unchanged, with the same 12 held-out fault buses—on data from higher-fidelity simulators, so only the ground-truth trajectories change. On the one-axis (flux-decay) model (a third-order per-machine model [1], Ch. 8) the retrained operator attains 38.6∘ pooled stable-trajectory RMSE (1.8∘ fault-on, 39.2∘ post-fault), 94.0% classification accuracy, a mean CCT error of 23.3 ms, and a conformal band of 121.3∘ half-width with 0.903 coverage (α=0.1). Adding a first-order automatic voltage regulator (a simplified IEEE DC1A exciter with KA=20, TA=0.2 s and rate limits) yields a fourth-order model on which the operator attains 25.4∘ pooled RMSE (2.2∘ fault-on, 25.5∘ post-fault), 94.0% accuracy, and conformal coverage 0.896 with half-width 127.7∘—the lowest absolute RMSE of the three generator models, largely because voltage regulation smooths the post-fault oscillation.

Adding uniform damping D=1 leaves the severity encoding unchanged—it is evaluated at the pre-fault equilibrium, where the damping term vanishes—so the same architecture is retrained on damped data. The fault-on region improves to 1.8∘ pooled RMSE, but the post-fault RMSE rises to 46.4∘ (versus 36.9​∘ undamped) and classification drops to 93.4% (CCT error 30.8 ms): near-boundary damped trajectories decay slowly, so small errors in the predicted decay rate accumulate over the 4 s window (maximum worst-case error 638∘). The split-conformal band remains approximately valid (mean coverage 0.90±0.02 over five calibration splits); damping does not simplify the near-boundary prediction task.

Conformal bands

The split-conformal band achieves empirical coverage close to nominal at every level on the Neva=178 evaluation scenarios: 91.6% (163/178) at the 95% target, 88.2% (157/178, CI 83–93%) at 90%, 67.4% (120/178) at 70%, and 43.8% (78/178) at 50% (Fig. 3); the same calibration on the multi-topology operators yields coverage of 0.915 on both held-out N−1 splits and 0.872/0.852 on N−2 buses and line pairs (half-widths 121–181∘). A Mondrian (grouped) calibration [23] by the post-fault peak of the pairwise spread—maxt≥tcmaxiδi(t)−miniδi(t) of the true trajectory, an offline diagnostic because the grouping uses ground-truth labels—shows, at α=0.1, that the half-width grows toward the stability boundary, from 76.1​∘ for scenarios peaking below 45∘ (group 48/51 calibration/evaluation, coverage 0.863), through 94.2∘ for 45–90∘ (44/52, 0.923), to 124.6​∘ above 90∘ (86/75, 0.800): coverage is approximately nominal away from the boundary and below nominal near it. The band is wide precisely where the stability decision is most critical; it is a conservative screening envelope, not a tight predictive interval.

The directly calibrated pairwise-difference score Rpw bounds the pairwise angle difference—the exact quantity in the stability criterion—to 148​∘ at α=0.1 (coverage 0.893), below the π threshold. Making a decision from this bound still requires the predicted spread to satisfy spread+qpw<π; on the evaluation half this rule resolves 0% of the predicted-stable scenarios at α=0.1 (rising to 31% at α=0.3 and 67% at α=0.5 as qpw tightens from 90.5∘ to 53.3∘), and the empirical wrong-release rate of the predicted-stable gate is 1.8% (2​ of 109), so any deployment must retain a TDS fallback for the unresolved fraction. Per-machine calibration with Bonferroni/Šidák corrections does not tighten the bound (pairwise 734∘), and calibrating on the validation set instead of held-out scenarios yields only 0.61 coverage at the nominal 0.90 target: the held-out-bus distribution is genuinely harder, so calibrating on held-out scenarios is necessary rather than a leakage concern. The calibration does not transfer across operating points (the N−0-calibrated interval covers only 0.51 of stable 0.8-load scenarios and 0.72 at 0.9 load), consistent with the operating-point limitation of Section 7. In the gated regime (band applied only after a predicted-stable decision, evaluated on the disjoint half): gate purity 96.6%, coverage over predicted-stable scenarios 0.914, and oracle-conditioned coverage 0.946 (56 scenarios; the last conditions on ground-truth labels and is not a deployment guarantee, since conditioning on the prediction breaks exchangeability). Alternative scores (amplitude-normalized 2.3%, first-swing 70.8∘, time-averaged 47.7∘; see the repository), adaptive/locally adaptive conformal methods [24], [25], and conformalized quantile regression [26] are candidates for reducing near-boundary conservatism and are left as future work.

Fig. 3.  Conformal band validity and adaptivity. Left: empirical coverage (with binomial intervals) against the target level α. Right: offline Mondrian diagnostic—the grouping uses the ground-truth post-fault peak of each scenario—shows the band half-width growing toward the stability boundary; bars are annotated with group size (calibration/evaluation) and per-group coverage.

The negative result: input encoding versus residual loss

The auxiliary physics-residual loss does not help in this setting: with the correctly scaled residual of [eq:physloss], the pooled stable-trajectory RMSE is 39.2∘ at λ=0.001 (versus 36.3​∘ for the data-only operator) and 76.5∘ at λ=0.01 (Table 2)—no improvement at the mildest weight and harmful at larger weights—and denser collocation does not help either (48.9∘ pooled, 92.9% accuracy). Without the correct post-fault initial condition the swing-equation residual is a weak constraint satisfied by many trajectories, so it adds regularization without information. This conclusion is scoped to the present system, protocol, weights, and architecture.

Near-boundary sensitivity and architecture ablations

Training on boundary-aware data (clearing times sampled within [0.4,1.2]×CCT per fault bus) raises the pooled post-fault RMSE from 36.9​∘ (uniform) to 70.8∘ (boundary-aware): near-boundary trajectories respond sharply to small parameter changes at the stability bifurcation, the regime the conformal band is designed to flag (the rise can reflect both intrinsic difficulty and the shifted sampling; we report the observation without attributing a single cause). Stratifying the test scenarios by their distance to the TDS CCT quantifies this: more than 100 ms inside the boundary the misclassification rate is 8.8%; within 30 ms it rises to 20.0% (n=60); and within 10 ms it reaches 46.2% (n=13, descriptive). The sharpest decisions are the least reliable. Replacing the branch network with self-attention does not improve accuracy (116.1∘ RMSE, 92.8% accuracy at p=512, versus 36.3​∘ and 95.4%), consistent with the severity encoding already capturing the machine-interaction structure, and a Fourier-neural-operator trunk likewise gives no improvement (47.4∘ RMSE, 92.6% accuracy). This supports the choice of a simple DeepONet with a physics-informed encoding.

Robustness to measurement noise

The encoding ai(k)=(Pmi−Peif)/Mi is itself computed from estimates of mechanical power, inertia, and fault-on electrical power, which carry measurement error. Perturbing the encoded acceleration vector with zero-mean Gaussian noise, the pooled stable RMSE rises from 36.3​∘ to 115.4∘ and accuracy drops from 95.4% to 89.1% at 5% noise (152.3∘/82.9% at 10%, 213.6∘/73.0% at 20%). The operator is therefore only moderately robust to feature noise in its input encoding; correlated or biased parameter-estimation errors and conformal coverage under input noise remain to be tested.

Scalability to the IEEE 118-bus system

To probe scalability beyond a single test system, we evaluate the operator on a custom dynamic test case built on the IEEE 118-bus static network: the 54 machine parameters (H, xd′) are taken from the grouped tables of Demetriou et al. [27] and assigned to pandapower’s case118 generators by rated capacity. No per-machine bus mapping and no per-unit base conversion are applied, so this is a synthetic dynamic case on the IEEE 118-bus network rather than a reproduction of the Demetriou system; results on it probe scalability of the method, not of any benchmark system. With 24 fault buses held out entirely from training (3760 training and 960 test scenarios), the retrained operator attains 92.9% classification accuracy and 121.9∘ stable-trajectory RMSE. The test set is 25.4% stable, so a majority-class baseline would score 74.6%; 92.9% therefore reflects real discrimination, close to the 39-bus 95.4% despite 5.4× more generators and 3.0× more buses. The higher trajectory RMSE reflects the larger state space and the much lower CCTs of the assigned 118-bus case (0.008–0.135 s), with a CCT error of 13.3 ms mean absolute (max 54.7 ms) on the 24 held-out buses. The split-conformal interval attains 0.918 coverage at α=0.1 with a half-width of 596.4∘—larger than the 180∘ stability threshold, so on the 118-bus case the interval validates the coverage mechanism but is vacuous for the stability decision; the directly calibrated pairwise score and richer calibration data are required before the bound is operationally useful at this scale.

Discussion and Limitations

First, the post-fault trajectory accuracy (36.9​∘ pooled RMSE) is limited and concentrated near the stability boundary, where the classical model response to small parameter changes is sharpest. Second, the conformal band is calibrated on stable scenarios and covers the finite output grid: it is a screening band for the stable subpopulation, not a stability certificate; near-boundary and unstable scenarios are conveyed through the classification, the wrong-release statistics of Section 6, and the heuristic CCT band rather than a numeric trajectory bound, and on the 118-bus case the per-machine band is currently vacuous for decisions. The π-in-4 s criterion is the operational label of this study; whether the first swing governs multi-swing instability in every scenario of richer models was not established here. Third, governors, power-system stabilizers, and dynamic loads are not modeled; the one-axis, exciter, and damped extensions retrain the same architecture and encoding rather than transferring weights, although the encoding itself needs no modification. Fourth, the pointwise MLP baseline is slightly worse than the operator on trajectory RMSE at full data (43.1∘ versus 36.3​∘ pooled), and the low-data advantage is indicative rather than established (Section 6.4); under the same conformal and CCT protocol the MLP is not worse, so the operator’s remaining advantages are the lower trajectory RMSE, continuous-time evaluation, and the low-rank structure, and certification tightness is not one of them. Fifth, the operator is trained at a fixed operating point and does not generalize to substantially different load levels: classification accuracy drops to 75% at 20% lower load, and training across multiple load levels does not resolve this (a model trained on four load levels attains only 65% on a held-out level). Deployment is therefore a screening tool within its calibrated operating domain; detecting and rejecting out-of-domain operating points (e.g., via the encoding norm or an OOD detector) is left as required future work, and operating-point generalization remains the binding obstacle to broader deployment. Sixth, only bolted three-phase faults at buses are modeled; parametric fault impedance and faults along line corridors are left to future work.

Conclusion

We presented a physics-encoded neural operator for transient stability assessment that predicts the full post-fault rotor-angle trajectory and the critical clearing time, with a split-conformal layer providing a finite-sample, distribution-free trajectory band for the stable subpopulation on the output grid. The key enabler is a physics-informed input encoding—the swing-equation acceleration vector, extended by a post-fault severity vector—which lets the operator generalize to unseen fault locations and to N−1 and N−2 line-trip topologies. On the IEEE 39-bus system the operator runs ≈214​× faster than time-domain simulation on identical hardware, classifies stability with 95.4% accuracy under the max-over-time π-separation criterion, and estimates the CCT to within 25.2 ms against an event-aligned reference; the directly calibrated pairwise bound (148​∘ at α=0.1) is coverage-valid, with a wrong-release rate and resolvable fraction reported in Section 6 and a TDS fallback for the remainder. A negative result shows that, for this system and protocol, the physics-informed input encoding is more effective than a physics-residual loss, and the encoding carries over unchanged to one-axis, exciter, and damped generator models and to a synthetic 118-bus dynamic case (92.9% accuracy), where trajectory-level bands remain vacuous for decisions. Together with the operating-point limitation, these results define a fast, uncertainty-aware screening tool for TSA within its calibrated domain, and a concrete agenda—out-of-domain detection, multi-swing and multi-operating-point calibration, and tighter near-boundary scores—for its deployment.

Acknowledgments

During the preparation of this work the authors used [TOOL NAME—e.g., Claude (Anthropic)] to polish the language and improve the readability of the manuscript. After using this tool, the authors reviewed and edited the content as needed and take full responsibility for the content of the publication.

Declarations

Funding. The authors declare no specific funding for this work.
https://github.com/La0Fe1/operator-transient-stability

CRediT authorship contribution statement. Zhenyu Liu: Conceptualization, Methodology, Software, Validation, Formal analysis, Investigation, Writing – original draft, Writing – review & editing. Jiale Wang: Validation, Writing – review & editing. Jiayong Liu: Validation, Writing – review & editing. Zhicheng Zeng: Validation, Writing – review & editing.

Data availability. The code reproducing all experiments—scenario generation, model training for all variants, conformal calibration, CCT binary search, and the sources for Figures 1–3 and Tables 1–3—is available at , together with the random seeds, software versions, and environment file.

Conflict of interest. The authors declare no competing interests.

Author biographies

Zhenyu Liu received ... [PLACEHOLDER: education background and research interests to be filled in by the authors].
Jiale Wang ... [PLACEHOLDER].
Jiayong Liu ... [PLACEHOLDER].
Zhicheng Zeng ... [PLACEHOLDER].

[1] 	P. W. Sauer and M. A. Pai, Power system dynamics and stability. Upper Saddle River, NJ: Prentice Hall, 1998.

[2] 	P. Kundur, Power system stability and control. New York: McGraw-Hill, 1994.

[3] 	B. Stott, “Power system dynamic response calculations,” Proceedings of the IEEE, vol. 67, no. 2, pp. 219–241, 1979.

[4] 	O. A. Alimi, K. Ouahada, and A. M. Abu-Mahfouz, “A review of machine learning approaches to power system security and stability,” IEEE Access, vol. 8, pp. 113512–113531, 2020.

[5] 	L. Lu, P. Jin, G. Pang, et al., “Learning nonlinear operators via DeepONet based on the universal approximation theorem of operators,” Nature Machine Intelligence, vol. 3, no. 3, pp. 218–229, 2021, doi: 10.1038/s42256-021-00302-5.

[6] 	J. Lei, M. G’Sell, A. Rinaldo, et al., “Distribution-free predictive inference for regression,” Journal of the American Statistical Association, vol. 113, no. 523, pp. 1094–1111, 2018.

[7] 	I. Karampinis, P. Ellinas, J. Vorwerk, et al., “Neural operators for power systems: A physics-informed framework for modeling power system components,” Electric Power Systems Research, vol. 263, p. 113755, 2027.

[8] 	A. Mollaali, G. Zufferey, G. Constante-Flores, et al., “Conformalized prediction of post-fault voltage trajectories using pre-trained and finetuned attention-driven neural operators,” Neural Networks, vol. 192, p. 107809, 2025, doi: 10.1016/j.neunet.2025.107809.

[9] 	G. Lu, J. Wang, S. Bu, et al., “Confidence-aware transient stability assessment using a large language model and conformal prediction,” in 2026 IEEE PES international meeting (PES IM), Hong Kong, China, 2026, pp. 1–5.

[10] 	Z. Li, N. Kovachki, K. Azizzadenesheli, et al., “Fourier neural operator for parametric partial differential equations,” in International conference on learning representations (ICLR), 2021. Available: https://arxiv.org/abs/2010.08895

[11] 	M. Raissi, P. Perdikaris, and G. E. Karniadakis, “Physics-informed neural networks: A deep learning framework for solving forward and inverse problems involving nonlinear partial differential equations,” Journal of Computational Physics, vol. 378, pp. 686–707, 2019, doi: 10.1016/j.jcp.2018.10.045.

[12] 	C. Moya, A. Mollaali, Z. Zhang, et al., “Conformalized-DeepONet: A distribution-free framework for uncertainty quantification in deep operator networks,” Physica D: Nonlinear Phenomena, vol. 471, p. 134418, 2025, doi: 10.1016/j.physd.2024.134418.

[13] 	B. Huang, S. Dong, and Q. Guo, “Optimal power flow of AC/DC power systems based on physics-informed neural networks,” in 2025 6th international conference on advanced electrical and energy systems (AEES), 2025, pp. 264–270.

[14] 	K. Chen, “Power flow analysis and optimal power flow with physics-informed deep learning,” PhD thesis, University of California, Santa Cruz, 2025.

[15] 	K. Chen, S. Liu, N. Yu, et al., “Distribution-aware graph representation learning for transient stability assessment of power system,” in 2022 international joint conference on neural networks (IJCNN), 2022, pp. 1–8. Available: https://arxiv.org/abs/2205.06576

[16] 	B. Hao, C. Hu, M. Zhong, et al., “Event-structured physics-informed neural networks for differentiable critical clearing boundaries,” arXiv preprint arXiv:2607.27681, 2026.

[17] 	J. Stiasny, G. S. Misyris, and S. Chatzivasileiadis, “Transient stability analysis with physics-informed neural networks,” arXiv preprint arXiv:2106.13638, 2021.

[18] 	X. Wang, Y. Xu, W. Gu, et al., “Fast critical clearing time calculation for power systems with synchronous and asynchronous generation,” arXiv preprint arXiv:2503.12132, 2025.

[19] 	C. Moya, S. Zhang, G. Lin, et al., “DeepONet-grid-UQ: A trustworthy deep operator framework for predicting the power grid’s post-fault trajectories,” Neurocomputing, vol. 535, pp. 166–182, 2023, doi: 10.1016/j.neucom.2023.03.015.

[20] 	Z. Ma, Z. Pitt, K. Azizzadenesheli, et al., “Calibrated uncertainty quantification for operator learning via conformal prediction,” Transactions on Machine Learning Research, 2024, Available: https://arxiv.org/abs/2402.01960

[21] 	T. Athay, R. Podmore, and S. Virmani, “A practical method for the direct analysis of transient stability,” IEEE Transactions on Power Apparatus and Systems, vol. PAS–98, no. 2, pp. 573–584, 1979.

[22] 	S. Wang, J. Yu, and W. Zhang, “Transient stability assessment using individual machine equal area criterion Part II: Stability margin,” IEEE Access, vol. 6, pp. 38693–38705, 2018, doi: 10.1109/ACCESS.2018.2853637.

[23] 	V. Vovk, A. Gammerman, and G. Shafer, Algorithmic learning in a random world. New York: Springer, 2005.

[24] 	I. Gibbs and E. J. Candes, “Adaptive conformal inference under distribution shift,” in Advances in neural information processing systems (NeurIPS), 2021, pp. 1660–1672.

[25] 	J. Lei and L. Wasserman, “Distribution-free prediction bands for non-parametric regression,” Journal of the Royal Statistical Society: Series B, vol. 76, no. 1, pp. 71–96, 2014.

[26] 	Y. Romano, E. Patterson, and E. J. Candes, “Conformalized quantile regression,” in Advances in neural information processing systems (NeurIPS), 2019, pp. 3543–3553.

[27] 	P. Demetriou, M. Asprou, J. Quirós-Tortós, et al., “Dynamic IEEE test systems for transient analysis,” IEEE Systems Journal, vol. 11, no. 4, pp. 2108–2117, 2017, doi: 10.1109/JSYST.2015.2444893.

∗Corresponding author. Email address: 3293857014@qq.com (Zhenyu Liu)